shrike fi pins and documentation

https://github.com/vicharak-in/shrike/blob/main/docs/PLATFORM_AGNOSTIC_FIRMWARE_GUIDE.md

firmware  links for shrike fi fpga and shrike lite fpga

https://github.com/vicharak-in/shrike/releases/

-------------------------------------------------------

install this shrike-fi-micropython.bin firmware for getting thonny ide support for burning firmware

https://github.com/vicharak-in/shrike/blob/main/docs/shrike_fi.md

python -m venv myenv

myenv\Scripts\activate

pip install esptool

python -m esptool --chip esp32s3 -b 460800 --before default_reset --after hard_reset write_flash --flash_mode dio --flash_size 4MB --flash_freq 80m 0x0 shrike-fi-micropython.bin

--------------------------------------------------------------------

In Thonny programming the fpga bin file (select micropython esp32)

Connect the Shrike-fi to your computer without holding the BOOT button.

Connect to the board in Thonny.

Open View → Files.

Find your FPGA_bitstream_MCU ( .bin ) file of the verilog code which is generated

Right-click the .bin → Upload to / so it is copied onto the Shrike-fi filesystem.

--------------------------------------------------------------------

Create a Python file containing and store it in shrikefi board

import shrike

shrike.flash("FPGA_bitstream_MCU ")

print("FPGA programmed")

---------------------------------------------------------------------